Tenaya

Engine Operation

- Check sea water sea cock  and open

- Check engine oil level

- Check transmit ion oil level

- Visual inspection of engine –

- Check cooling water level

- Start engine

- Check for water in exhaust

- Warm for 5 minutes at low speed

- Visual check of engine for leaks, etc.

- Check warning lights

- When underway, raise speed slowly.  At least 10 min. to max speed

- Check exhaust gas color

- Check water temperature and oil pressure

- While underway check water temp & oil pressure at least once each hour

- Check engine compartment at least every 2 hours

- When stopping, operate engine at 650-700 RPMs for 5 minutes

- Raise speed to 1800 RPMs just before stopping the engine and then idle for 3-4 seconds to prevent carbon build-up

- After stopped and engine cool check for oil and water leaks and for loose nuts and bolts.